Glossaries

| Part | Meaning |
| --- | --- |
| BARAKUDA | Field name. The well targets reservoirs below the Tapis field. |
| -1 | Well number 1. |

| Abbreviation | Meaning |
| --- | --- |
| A |  |
| AFE | Authorization for Expenditure – Approved budget and plan for the well. |
| AMSL | Above Mean Sea Level – Height above average sea level. |
| API | American Petroleum Institute – Standards body; used in mud tests and thread types. |
| assy | Assembly – A set of connected tools or parts. |
| Avg. | Average |
| Azi. | Azimuth – Direction of the well path from north, in degrees. |
| B |  |
| BA | Bearing Assembly – Bearing and seal unit of the RCD. (to be confirmed) |
| BART | Bearing Assembly Running Tool – Tool used to install or pull the BA. (to be confirmed) |
| bbl | Barrel – Volume unit; 1 bbl = 158.987 litres. |
| BG | Background Gas – Normal gas level in the mud while drilling. |
| BGOR | Bearing, Gauge, Other, Reason – Last four columns of the IADC bit dull grade. |
| BHA | Bottom Hole Assembly – Tools at the bottom of the drill string (bit, motor, MWD). |
| BMP | Unknown – Appears as '19.4 BMP' in the DGOS mud weight field. (to be confirmed) |
| B/O | Break Out – Unscrew a pipe or tool connection. |
| BOP | Blowout Preventer – Valves used to close in the well. |
| btm | Bottom |
| C |  |
| C1 | Methane – Lightest hydrocarbon gas. |
| C2 | Ethane |
| C3 | Propane |
| CaCl2 | Calcium Chloride – Salt used in synthetic-based mud. |
| CBU | Circulate Bottoms Up – Pump until mud from the bottom reaches surface. |
| CCL | Casing Collar Locator – Detects casing joints to correlate depth. |
| CCS | Continuous Circulation System – Keeps mud circulating during pipe connections. (to be confirmed) |
| CDS | Casing Drive System – Rotates casing using the top drive. |
| CH | Cased Hole – Part of the well already lined with casing. |
| cmt | Cement |
| C/O | Change Out – Replace one component with another. |
| CO2 | Carbon Dioxide |
| COB | Unknown – Value in the DGOS 'Operatorship' field. (to be confirmed) |
| cp | Centipoise – Viscosity unit. |
| CRT | Casing Running Tool – Runs casing using the top drive. |
| csg | Casing – Steel pipe lining the well. |
| CSS | Unknown – Seen as 'CSS subs' on drill pipe in the DDR. (to be confirmed) |
| CWD | Casing While Drilling – Drilling with the casing itself. |
| D |  |
| D13 / D18 | Phase codes – Phase codes in the DDR operation table. (to be confirmed) |
| DC | Drill Collar – Heavy pipe above the bit that adds weight. |
| DD | Directional Drilling – Service that steers the well path. |
| DDR | Daily Drilling Report – Daily report of rig operations (PDF title: Daily Operation Report). |
| DFE | Drill Floor Elevation – Height of the drill floor above sea level. |
| DFS | Days From Spud – Days since drilling of the well started. |
| DGOS | Daily Geological Operations Summary – Daily geology and operations report. |
| displ | Displace – Replace the fluid in the well with another fluid. |
| Dleg | Dogleg (Severity) – Rate of change of well direction, °/30 m. |
| DOL | Days On Location – Days the rig has been at the well site. |
| DP | Drill Pipe – Main pipe of the drill string. |
| drlg | Drilling |
| DS | Drilling Supervisor – Operator's supervisor on the rig. |
| DSI | Dipole Shear Sonic Imager – SLB sonic logging tool. |
| DSI-HT | DSI High Temperature – High-temperature version of the DSI tool. (to be confirmed) |
| E |  |
| ea | Each |
| ECD | Equivalent Circulating Density – Effective mud density while pumping. |
| EFF | Efficiency – Mud pump efficiency. |
| EMW | Equivalent Mud Weight – Pressure expressed as a mud weight (ppg). |
| Est. | Estimated |
| E/W | East/West – Horizontal offset east or west of the reference. |
| F |  |
| f/ | From |
| FC | Filter Cake – Mud layer left on the hole wall. |
| FIT | Formation Integrity Test – Pressure test below the shoe to a set limit. |
| FLID | Fluid Identification – MDT station used to identify formation fluid. (to be confirmed) |
| FLW | Activity code – Used on the flow check row; likely 'flow check'. (to be confirmed) |
| fm | Formation – A rock layer. |
| FRM | Phase code – Used during logging; likely 'formation evaluation'. (to be confirmed) |
| FS | Float Shoe – Casing shoe with a one-way valve. |
| ft | Feet – 1 ft = 0.3048 m. |
| FTD | Final Total Depth – Planned final depth of the well. (to be confirmed) |
| G |  |
| GR | Gamma Ray – Log of natural rock radioactivity. |
| H |  |
| H2S | Hydrogen Sulphide – Toxic gas. |
| HGS | High Gravity Solids – Heavy solids in the mud, mainly barite. |
| HSE | Health, Safety and Environment |
| HT | High Temperature |
| HTHP | High Temperature High Pressure – Mud fluid-loss test condition. |
| HWDP | Heavy Weight Drill Pipe – Pipe between drill collars and drill pipe. |
| I |  |
| IADC | International Association of Drilling Contractors – Owner of the bit dull grading standard. |
| iC4 / nC4 | Iso-butane / Normal butane – Butane gas components. |
| iC5 / nC5 | Iso-pentane / Normal pentane – Pentane gas components. |
| iCWD | Intelligent Circulation While Drilling – Smart circulation system used while drilling. |
| ID | Inner Diameter |
| ILT | Invisible Lost Time – Lost time that is not recorded as NPT. |
| in | Inch – 1 in = 25.4 mm. |
| Inc. | Inclination – Angle of the well from vertical. |
| IODL | Inner, Outer, Dull, Location – First four columns of the IADC bit dull grade. |
| J |  |
| JSA | Job Safety Analysis – Hazard review before a job; also an activity code in the DDR. |
| jt | Joint – One length of pipe. |
| K |  |
| K+ | Potassium |
| kip | Kilopound – 1 kip = 1,000 lbf. |
| L |  |
| LAS | Log ASCII Standard – Text file format for well log data. |
| lbf / lbm | Pound-force / Pound-mass |
| lbs | Pounds – Written 'Ibs' by mistake in the DDR. |
| L/D | Lay Down – Put pipe or tools down, out of use. |
| LGS | Low Gravity Solids – Light solids in the mud, mainly drilled cuttings. |
| LOH | Activity code – Used on wireline rows; likely 'logging open hole'. (to be confirmed) |
| LOT | Leak Off Test – Pressure test until the formation starts taking fluid. |
| LTA | Lost Time Accident – Accident causing lost work days. |
| LWD | Logging While Drilling – Log measurements taken while drilling. |
| M |  |
| MAASP | Maximum Allowable Annular Surface Pressure – Highest safe annulus pressure at surface. |
| MBT | Methylene Blue Test – Measures reactive clay in the mud. |
| MD | Measured Depth – Length along the well path. |
| MDDF | Measured Depth from Drill Floor – MD measured from the drill floor. |
| MDT | Modular Formation Dynamics Tester – SLB tool for formation pressure and fluid samples. |
| MP | Mud Pump |
| MPD | Managed Pressure Drilling – Drilling with controlled annulus pressure. |
| MPSR | Unknown – Type of MDT sample bottle. (to be confirmed) |
| MT | Metric Ton |
| MTM | Metal To Metal – Type of seal. |
| M/U | Make Up – Screw together a pipe or tool connection. |
| MUSD | Million US Dollars |
| MW | Mud Weight – Density of the drilling mud (ppg). |
| MWD | Measurement While Drilling – Direction and drilling data sent while drilling. |
| N |  |
| NaCl | Sodium Chloride – Salt. |
| nC6 / nC7 / nC8 | Hexane / Heptane / Octane – Heavier gas components. |
| N/D | Nipple Down – Remove equipment from the wellhead. |
| NIL | None – Nothing to report. |
| NLTA | Unknown – Safety statistic in the DDR. (to be confirmed) |
| NOV | National Oilwell Varco – Equipment and MPD service company. |
| NPT | Non-Productive Time – Time lost to unplanned problems. |
| N/S | North/South – Horizontal offset north or south of the reference. |
| O |  |
| OBM | Oil Based Mud |
| OD | Outer Diameter |
| OH | Open Hole – Part of the well without casing. |
| OPRN | Operation / Operating – Code used for productive status and rig status. (to be confirmed) |
| OWR | Oil-Water Ratio – Oil to water ratio in the mud (e.g. 80/20). |
| P |  |
| PBR | Polished Bore Receptacle – Smooth sleeve that seals a liner or tubing. |
| PCSB | PETRONAS Carigali Sdn. Bhd. – Operator named in the DGOS. |
| PDC | Polycrystalline Diamond Compact – Bit type with synthetic diamond cutters. |
| PDG | Permanent Downhole Gauge – Fixed pressure/temperature sensor in the well. |
| PEX | Platform Express – SLB combined logging service. |
| Pf / Mf / Pm | Filtrate and mud alkalinity – Mud chemistry measurements. |
| PH | Pilot Hole – Small first hole drilled to check the ground. |
| PJSM | Pre-Job Safety Meeting – Short safety talk before a job. |
| PM | Peninsular Malaysia – Region in the well data. |
| PMCD | Pressurized Mud Cap Drilling – Drilling technique used during total mud losses. |
| POB | Personnel On Board – Number of people on the rig. |
| POH / POOH | Pull Out Of Hole – Pull the string out of the well. |
| POH1 | Activity code – Used on POOH rows in the DDR. (to be confirmed) |
| ppg | Pounds Per Gallon – Mud density unit. |
| ppm | Parts Per Million |
| psi | Pounds per Square Inch – Pressure unit. |
| PTT | PTT Public Company Limited – Name in the DDR header. |
| P/U | Pick Up – Lift pipe or tools. |
| PV | Plastic Viscosity – Mud property (cp). |
| Q |  |
| QAIT | Array induction tool variant – SLB resistivity tool. (to be confirmed) |
| QS | Quicksilver Probe – SLB MDT sampling probe. (to be confirmed) |
| R |  |
| RA | Radioactive – As in 'RA source' in logging tools. |
| R/B | Rack Back – Stand pipe upright in the derrick. |
| RCD | Rotating Control Device – Rotating seal above the BOP, used for MPD. |
| R/D | Rig Down – Remove equipment. |
| RDR | Re-Drill – In a well name, e.g. 1RDR1 = well 1, re-drill no. 1. (to be confirmed) |
| Reg | Regular – API regular thread type. |
| RIH | Run In Hole – Lower the string into the well. |
| RM | Recorded Mode – LWD data downloaded from tool memory. |
| ROP | Rate Of Penetration – Drilling speed (m/hr). |
| RPM | Revolutions Per Minute – Rotation speed. |
| RSS | Rotary Steerable System – Steers the well while the string rotates. |
| RT | Running Tool / Real Time – Running Tool in the report glossary; Real Time in WellXP 'RT Logs'. |
| R/U | Rig Up – Install equipment. |
| S |  |
| SBM | Synthetic Based Mud – Mud made with synthetic oil. |
| SID | Unknown – Document folder name in WellXP. (to be confirmed) |
| SLB | Schlumberger – Oilfield service company (wireline, logging). |
| S/O | Slack Off – Lower the string; also the weight when lowering. |
| SPM | Strokes Per Minute – Mud pump speed. |
| SPP | Stand Pipe Pressure – Mud pump pressure at the standpipe. |
| STDBY | Standby – Rig status while waiting on another service. |
| stds | Stands – Several joints of pipe stored as one unit. |
| STOP | Safety Training Observation Program – Safety observation card system. (to be confirmed) |
| T |  |
| tbg | Tubing – Production pipe. |
| TD | Total Depth – Final depth of the well or hole section. |
| TDS | Top Drive System – Rotates and lifts the string from above. |
| TFA | Total Flow Area – Combined area of the bit nozzles. |
| TMD | Total Measured Depth |
| TPR | Top Pipe Ram – Upper BOP ram that closes around pipe. |
| TRS | Tubular Running Services – Service that runs casing and tubing. (to be confirmed) |
| t/shoot | Troubleshoot – Find and fix a problem. |
| TT | Trip Tank – Small tank used to monitor the hole while tripping. |
| TVD | True Vertical Depth – Straight-down depth. |
| TVDDF | True Vertical Depth from Drill Floor |
| TVDSS | True Vertical Depth Sub-Sea – Vertical depth below sea level. |
| U |  |
| UR | Underreamer – Tool that enlarges the hole. |
| UTM | Universal Transverse Mercator – Map coordinate system. |
| V |  |
| V. sec. | Vertical Section – Horizontal distance along a chosen direction. |
| W |  |
| w/ | With |
| WB | Wear Bushing – Protects the wellhead bore from wear. |
| WBM | Water Based Mud |
| WD | Water Depth – Sea depth at the well location. |
| WGS 84 | World Geodetic System 1984 – Coordinate datum. |
| WH | Wellhead |
| WITSML | Wellsite Information Transfer Standard Markup Language – Standard for real-time well data. |
| WL | Wireline – Tools run into the well on a cable. |
| WLD | Wireline Depth – Depth measured by the wireline cable. |
| WOB | Weight On Bit – Load on the bit. |
| WOW | Waiting On Weather – Operations stopped due to bad weather. |
| WSG | Wellsite Geologist – Geologist working on the rig. |
| X |  |
| XLR | XL-Rock – SLB sidewall coring service. (to be confirmed) |
| X/O | Cross Over – Adapter between different thread types. |
| Y |  |
| YP | Yield Point – Mud property (lbf/100ft²). |